Xin Wen-  Distributed Feedback Laser modeling

- Set Bragg wavelength λbragg=1315nm, Cavity length=500μm, naverage=3, gain start with [4000, 10000]/m, decreases when κL increases.   Pick κL =[0.5, 1.0, 2.0, 3.5, 5] to plot gain vs. wavelength. κL value is labeled on graph.

- Set wavelength start from 1305nm, increase with step 0.01, end at 1325nm. Calculate the following parameters

κ=κLL

Λ=λbragg2naverage  (grating pitch)

β=2πnaverageλ

Δβ=β-πΛ

- Set gain g start with 10, increase with step 10, end at 4000. For each g, S can be calculated by  S is complex propagation constant  g is gain

.

Thus for a given g, S is a function of ∆β.

- For each element in S(∆β), calculate the equation

- Based on the result in step 4, take κL =1 and gain= 6000/m for example. The points closed to 0 is the working wavelength for this gain.

- Plot gain vs. wavelength

- Repeat 4-6 for different gain.

- Repeat 2-7 for different κL

Note: The top graph for each κL has relative loose criteria (closed to 0 in step 5), and the bottom graph have  relative strict criteria.